1.Identification of the substance/preparation and the company

1.1	Identification of the substance or preparation:

Synonyms :	-

For 1-propanol:

CAS-no.	:	000071-23-8

EEC index no.	:	603-003-00-0	NFPA code	:	1-3-0

EINECS no.	:	200-746-9	Molecular weight	:	60.1

RTECS no.	:	UH8225000	Formula	:	C3H8O

1.2	Company/undertaking identification:

Indigo Electronic Printing Systems Ltd.

PO Box 150, Rehovot, 76101

Israel

Tel.:  +972-8-938-1818

1.3	Telephone number for emergency:

+972-8-938-1818

2.Composition/information on ingredients

Hazardous ingredients	CAS no.	Conc	 Hazard  	Risks

in %	 class.	(R-phrases)

1-Propanol	000071-23-8	 > 90	  F	11

3.Hazards identification

-	Highly flammable

-	Gas/vapour flammable with air within explosion limits

-	May build up electrostatic charges: risk of ignition hazard

-	Gas/vapour spreads at floor level: ignition hazard

- May be narcotic if swallowed or inhaled

4.First aid measures

4.1	Eye contact:

-	Rinse immediately with plenty of water for 15 minutes

-	Do not apply neutralizing agents

-	Consult a doctor/medical service if irritation persists

4.2	Skin contact:

-	Wash immediately with lots of water and soap for 15 minutes

-	Remove clothing before washing

-	Do not remove clothing if it sticks to the skin

-	Consult a doctor/medical service if irritation persists

4.3	After inhalation:

-	Unconscious: maintain adequate airway and respiration

-	Remove the victim into fresh air

-	Consult medical service if breathing problems develop

4.4	After ingestion:

-	Never give water to an unconscious person

-	Do not allow vomiting

-	Doctor: gastric lavage

-	Consult a doctor/medical service if you feel unwell

5.Fire-fighting measures

5.1	Suitable extinguishing media:

-	Water spray

-	Alcohol foam

-	BC powder

-	Carbon dioxide

5.2	Unsuitable extinguishing media:

-	Solid water jet ineffective as extinguishing medium

5.3	Special exposure hazards:

-	On burning : release of harmful gases/vapours

(carbon monoxide - carbon dioxide)

5.4	Instructions:

-	Cool tanks/drums with water spray/remove them into safety

-	Do not move the load if exposed to heat

5.5	Special protective equipment for firefighters:

-	Compressed air/oxygen apparatus

-	Protective clothing for exposure to chemicals

6.Accidental release measures

6.1	Personal protection: see 8.3

6.2	Environmental precautions:

-	Substance must not be discharged into the sewer

-	Collect/pump leaking substance into suitable containers

-	Plug the leak, cut off the supply

-	Dam up the liquid spill

-	Try to reduce evaporation

6.3	Clean-up:

-	Damaged/cooled tanks must be emptied

-	Do not use compressed air for pumping over spills

-	Take up liquid spill into adsorbent material (dry sand,earth,vermiculite)

-	Scoop adsorbed substance into closing containers

-	Carefully collect the spill/leftovers

-	Take collected spill to manufacturer/competent authority

7.Handling and storage

7.1	Handling:

-	Reduce/avoid exposure and/or contact

-	Keep container tightly closed

-	Use spark-/explosionproof appliances and lighting system

-	Take precautions against electrostatic charges

-	Handle uncleaned empty containers as full ones

7.2	Storage:

-	Store in a cool area

-	Ventilation at floor level

-	Keep away from: heat and ignition sources, oxidizing agents,

acids and halogens

Storage temperature: 20 °C

7.3	Materials for packaging:

- suitable	:	steel, stainless steel, aluminium, iron, copper, glass

8.Exposure controls/Personal protection

8.1	Recommended engineering controls:

-	Measure the concentration in the air regularly

-	Work under local exhaust/ventilation

8.2	Exposure limits:

For 1-propanol:

TLV value:

TLV TWA	:	200 ppm	492 mg/m3  (ACGIH ´96)

TLV STEL	:	250 ppm	614 mg/m3  (ACGIH ´96)

8.3	Personal protection:

eye protection:

-	Protective goggles

skin protection:

-	Gloves

-	Protective clothing

materials for protective clothing:

-	butyl rubber

-	viton

respiratory protection:

-	Gas mask with filter type A

9.Physical and chemical properties

For 1-propanol:

9.1	Appearance (at 20°C) 	:	Liquid

9.2	Odour	:	Characteristic

9.3	Colour	:	Yellow

9.4	pH value	:	7

9.5	Boiling point/boiling range	:	97	°C

9.6	Melting point/melting range	:	-126	°C

9.7	Flashpoint	:	15	°C

9.8	Auto-ignition point	:	412	°C

9.9	Explosion limits	:	2/14	vol%

9.10	Vapour pressure (at 20°C)	:	19	hPa

9.11	Relative density (at 20°C)	:	0.8

9.12	Water solubility	:	Completely

9.13	Soluble in	:	Ethanol, ether, acetone, propyleneglycol

9.14	Relative vapour density	:	2.1

9.15	Saturation concentration	:	46	g/m3

9.16	Viscosity	:	0.0023	Pa.s

10.Stability and reactivity

en reactiviteit

10.1	Stability:

-	Stable under normal conditions

10.2	Reactivity:

-	Reacts violently with (strong) oxidizers and (some) acids

with (increased) risk of fire/explosion

-	On burning: release of harmful gases/vapours

(carbon monoxide - carbon dioxide)

11.Toxicological information

11.1	Acute toxicity:

For 1-propanol:

LD50 oral rat	:	> 2000	mg/kg

LD50 dermal rat	:	5040	mg/kg

LC50 inhalation rat	:	N.D.	mg/l/4 h

11.2	Chronic toxicity:

EEC carc. cat.:  not listed

EEC muta. cat.:  not listed

EEC repr. cat.:  not listed

Carcinogenicity (TLV):  not listed

IARC classification:  not listed

11.3	Routes of exposure:	swallowed, inhalation, eyes and skin

11.4	Acute effects/symptoms:

- Substance is absorbed through the skin

- Slightly irritant to skin, eyes and respiratory organs

- May be narcotic if swallowed or inhaled

- CNS depression

- Dizziness

- Headache

- Risk of aspiration pneumonia

- Vomiting

- Nausea

- Abdominal pain

- Diarrhoea

EXPOSURE TO HIGH CONCENTRATIONS:

- Irritation of the respiratory tract/coughing

- Irritation/redness of the eye tissue

- Coordination disorders

- Disturbances of consciousness

- Respiratory difficulties

11.5	Chronic effects:

ON CONTINUOUS/REPEATED EXPOSURE/CONTACT:

- Irritation of the skin

- Skin rash/inflammation

12.Ecological information

For 1-propanol:

12.1	Biodegradation:

- soil:	T ½	:	N.D.	days

BOD5	:	0.47/1.63	g O2/g substance

COD	:	2.23	g O2/g substance

- water:	-	Readily biodegradable (test: 64%, 5 days, OECD 301D)

12.2	Bioaccumulation:	- log Pow	:	0.25/0.34

- BCF	:	N.D.

12.3	Aquatic toxicity:

-	LC50 (96 h) :    4480 mg/l (PIMEPHALES PROMELAS)

-	LC50 (48 h) :    3025 mg/l (DAPHNIA PULEX)

-	LC50 (48 h) :    2500 mg/l (ASELLUS SP.)

12.4	Other information:

- WGK: 1

- Effect on the ozone layer	:	N.D.

- Waste water purification	:

Nitrification of activated sludge is inhibited at 20 g/l, 50%, 2 hours

13.Waste disposal considerations

-	Hazardous waste (91/689/EEC)

-	Recycle by distillation

-	Incinerate under surveillance

-	Do not discharge into surface water

-	Obtain the consent of pollution control authorities before discharging

to wastewater treatment plants

14.Transport information

33

1274

14.1	Substance identification number (UN number): 1274

14.2	Transport by road/rail (ADR/RID): class 3, 3b)

Danger code: 33

14.3	Maritime transport (IMDG code): class 3.2  p. 3272-1

EMS	:	3-06

MFAG	:	305

Marine pollutant	:	-

14.4	Inland navigation (ADNR): class 3, 3b)

14.5	Air freight (ICAO): class 3

Instruction “passenger”: 305/Y305

Instruction “cargo”: 307

14.6	Other information: UN 1274 n-Propanol

15.Regulatory information

Labelling in accordance with EC directives 67/548/EEC and 88/379/EEC

Highly flammable

R11         :	Highly flammable

S(02)       :	(Keep out of reach of children)

S07         :	Keep container tightly closed

S16         :	Keep away from sources of ignition - No smoking

Max VOC content  760 g/l

16.Other information

The information provided on this MSDS is correct to the best of our knowledge, information and belief at the date of its publication.  The information given is designed only as a guidance for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered as a warranty or quality specification.  The information relates only to the specific material designated and may not be valid for such material used in combination with any other material or in any process, unless specified in the text.

N.A.	= NOT APPLICABLE

N.D.	= NOT DETERMINED

MSDS established	:	05-09-1997

Reference number	:	BIG\10242.1